# Automix Engine Implementation Guide

### For AI-Powered Music Applications

## PREFACE

This document serves as a comprehensive engineering guideline for building an Automix engine that replicates the behavior of Apple Music’s DJ-style crossfade system. The goal is to create seamless, intelligent, beat-aware, harmonically conscious transitions between consecutive tracks in a streaming or playback environment.

Your system will operate In three phases:

1. **Pre-Analysis Pipeline** (happens before/during buffering)

2. **Transition Decision Engine** (happens when approaching the end of a track)

3. **Real-Time Audio Processing** (happens during the actual crossfade window)

## PHASE 1: PRE-ANALYSIS PIPELINE

### 1.1 — Beat and Tempo Detection

**Objective:** Extract a precise BPM value and generate a beat grid (an array of timestamps marking every beat in the track).

**Instructions:**

- Use an onset detection algorithm. Recommended approaches include:

- Spectral flux analysis (monitor energy changes across frequency bins over time)

- A trained neural network beat tracker (e.g., `madmom`, `librosa.beat`, or a custom Core ML / TensorFlow Lite model)

- Detect both the **global BPM** (a single float, e.g., `121.3`) and a **beat grid** (an ordered array of millisecond timestamps for every detected beat)

- Quantize the beat grid so that beats are evenly spaced according to the detected BPM. Small timing deviations from live recordings should be smoothed unless the track has intentional tempo changes

- Identify the **downbeat** (beat 1 of each bar). Most western pop/electronic music is in 4/4 time, so every 4th beat is a downbeat. Your downbeat detector should confirm this by analyzing kick drum patterns and harmonic rhythm changes

- Store the result as metadata attached to the track:

```json

{

“track_id”: “abc123”,

“bpm”: 121.3,

“time_signature”: “4/4”,

“beat_grid”: [0, 496, 991, 1487, 1983, …],

“downbeats”: [0, 1983, 3966, 5949, …],

“tempo_stable”: true

}

```

**Edge Cases to Handle:**

- If a track has a tempo change (e.g., a ballad that accelerates into a chorus), flag `tempo_stable: false` and store a tempo map (array of BPM values per section)

- If the track is rubato (no fixed tempo, e.g., classical piano), flag the track as `not_mixable` and fall back to a simple volume crossfade

- If BPM detection confidence is below 70%, fall back to simple crossfade

### 1.2 — Musical Key Detection

**Objective:** Determine the key and scale (major/minor) of the track so you can assess harmonic compatibility between consecutive songs.

**Instructions:**

- Use a chromagram-based key detection algorithm:

1. Compute a Short-Time Fourier Transform (STFT) of the full audio signal

2. Map the resulting frequency bins to the 12 pitch classes (C, C#, D, D#, E, F, F#, G, G#, A, A#, B)

3. Build a chroma vector (a 12-dimensional vector representing the energy of each pitch class)

4. Correlate the chroma vector against the 24 Krumhansl-Kessler key profiles (12 major + 12 minor)

5. The key profile with the highest correlation score is the detected key

- Map the result to the **Camelot Wheel** notation for easy compatibility checking later:

```

Key Mapping Examples:

C Major  → 8B     A Minor  → 8A

G Major  → 9B     E Minor  → 9A

D Major  → 10B    B Minor  → 10A

…

```

- Store the result:

```json

{

“track_id”: “abc123”,

“key”: “A Minor”,

“camelot”: “8A”,

“key_confidence”: 0.87

}

```

**Compatibility Rules (Camelot Wheel):**

Two tracks are harmonically compatible if their Camelot codes satisfy any of the following:

- **Same code** (e.g., 8A → 8A) — Perfect match

- **Adjacent number, same letter** (e.g., 8A → 7A or 8A → 9A) — Compatible (up/down a fifth)

- **Same number, different letter** (e.g., 8A → 8B) — Relative major/minor switch

- **Everything else** — Potentially clashing; use a short transition strategy

### 1.3 — Structural Segmentation

**Objective:** Identify the internal structure of the track — intro, verse, chorus, bridge, outro, instrumental breaks, drops, and silence.

**Instructions:**

- Use a self-similarity matrix approach:

1. Compute MFCCs (Mel-Frequency Cepstral Coefficients) or chroma features for every short frame (~100ms) of audio

2. Build a self-similarity matrix by computing the cosine distance between every pair of frames

3. Apply checkerboard kernel convolution along the diagonal to detect boundaries where the texture of the music changes

4. Cluster the resulting segments by similarity (segments that sound alike are likely repeated sections like choruses)

- Alternatively, use a pre-trained neural segmentation model (e.g., trained on the SALAMI dataset)

- Label segments with structural roles. At minimum, you need to identify:

- **Intro** — The opening section before the first vocal or main melody enters

- **Outro** — The closing section after the final chorus/vocal

- **Last vocal moment** — The exact timestamp where the last sung word ends

- **Drops / Builds** — Sudden energy increases (critical for EDM genres)

- **Silence / Fade** — Trailing silence at the end of a track (must be trimmed)

- Store as:

```json

{

“track_id”: “abc123”,

“segments”: [

{“label”: “intro”,   “start_ms”: 0,      “end_ms”: 15200},

{“label”: “verse”,   “start_ms”: 15200,  “end_ms”: 45600},

{“label”: “chorus”,  “start_ms”: 45600,  “end_ms”: 68400},

{“label”: “verse”,   “start_ms”: 68400,  “end_ms”: 98800},

{“label”: “chorus”,  “start_ms”: 98800,  “end_ms”: 121600},

{“label”: “bridge”,  “start_ms”: 121600, “end_ms”: 136800},

{“label”: “chorus”,  “start_ms”: 136800, “end_ms”: 159600},

{“label”: “outro”,   “start_ms”: 159600, “end_ms”: 189200}

],

“last_vocal_ms”: 162300,

“trailing_silence_ms”: 187000

}

```

### 1.4 — Loudness and Energy Profiling

**Objective:** Measure the perceived loudness (in LUFS) and spectral energy profile across the track so you can manage gain during transitions.

**Instructions:**

- Compute **integrated LUFS** (Loudness Units Full Scale) per the ITU-R BS.1770-4 standard for the entire track

- Compute **short-term LUFS** (3-second window) for each segment identified in step 1.3

- Compute a **spectral energy curve** — split the frequency spectrum into 3 bands:

- **Low:** 20Hz – 200Hz (bass, kick drums, sub)

- **Mid:** 200Hz – 4kHz (vocals, guitars, snares, leads)

- **High:** 4kHz – 20kHz (hi-hats, cymbals, air, sibilance)

- Measure average RMS energy per band per segment

- Store as:

```json

{

“track_id”: “abc123”,

“integrated_lufs”: -14.2,

“segment_loudness”: [

{“label”: “intro”,  “lufs”: -18.1, “low_rms”: 0.02, “mid_rms”: 0.08, “high_rms”: 0.04},

{“label”: “chorus”, “lufs”: -12.5, “low_rms”: 0.15, “mid_rms”: 0.22, “high_rms”: 0.11},

{“label”: “outro”,  “lufs”: -16.8, “low_rms”: 0.06, “mid_rms”: 0.10, “high_rms”: 0.05}

]

}

```

### 1.5 — Vocal Activity Detection

**Objective:** Identify exactly where vocals are present so you never overlap two vocal sections.

**Instructions:**

- Run a vocal activity detection (VAD) model on the audio. You can use:

- A source separation model (e.g., Demucs, Spleeter) to isolate the vocal stem, then measure its RMS energy over time

- A binary classifier trained on vocal vs. non-vocal frames

- Produce a **vocal activity timeline**: an array of boolean flags per beat or per 500ms frame indicating whether vocals are active

```json

{

“track_id”: “abc123”,

“vocal_activity”: [

{“start_ms”: 15200, “end_ms”: 45100, “active”: true},

{“start_ms”: 45100, “end_ms”: 45600, “active”: false},

{“start_ms”: 45600, “end_ms”: 66200, “active”: true},

…

],

“last_vocal_end_ms”: 162300

}

```

## PHASE 2: TRANSITION DECISION ENGINE

This phase fires when the currently playing track (Track A) approaches its final segments. The engine must decide **when**, **how long**, and **what type** of transition to execute.

### 2.1 — Determine the Mix-Out Point of Track A

**Instructions:**

- Look at Track A’s structural segmentation data

- Apply the following priority logic:

```

PRIORITY 1: If Track A has a labeled “outro” segment:

→ Set mix_out_point = start of outro

→ Available blend window = duration of outro

PRIORITY 2: If Track A has no outro but has trailing silence:

→ Set mix_out_point = last_vocal_end_ms

→ Available blend window = (trailing_silence_ms – last_vocal_end_ms)

PRIORITY 3: If Track A has no outro and no trailing silence:

→ Set mix_out_point = start of the last 8-bar phrase

→ Apply a forced echo-out or reverb tail to create artificial space

→ Available blend window = 8 bars (calculate in ms from BPM)

FALLBACK: If structural analysis failed:

→ Set mix_out_point = (track_duration – 8000ms)

→ Use a simple 8-second volume crossfade

```

- **Phrase-lock the mix_out_point:** Snap it to the nearest downbeat in the beat grid. Never start a transition mid-bar.

### 2.2 — Determine the Mix-In Point of Track B

**Instructions:**

- Look at Track B’s structural segmentation data

- Apply the following logic:

```

PRIORITY 1: If Track B has a labeled “intro” segment:

→ Set mix_in_point = start of intro (typically 0ms or after any silence)

→ Blend Track B’s intro over Track A’s outro

PRIORITY 2: If Track B has no intro (starts directly with vocals/chorus):

→ Set mix_in_point = beat 1 of the track

→ Shorten the blend window: use a quick 2-4 beat “drop mix” instead of a gradual blend

→ Apply reverb/echo tail to Track A to fill the gap

PRIORITY 3: If Track B starts with silence:

→ Trim the silence

→ Set mix_in_point = first detected onset (first audible sound)

```

### 2.3 — Calculate the Blend Duration

**Instructions:**

- The blend duration is the overlap period where both tracks play simultaneously

- Base it on the shorter of:

- Track A’s available outro/exit window

- Track B’s available intro/entrance window

- Apply these constraints:

```

MINIMUM blend duration: 2 beats (prevents abrupt jarring cuts)

MAXIMUM blend duration: 32 bars (prevents excessively long overlaps that fatigue the listener)

IDEAL blend duration: 8–16 bars for songs with compatible tempos and keys

```

- Round the blend duration to the nearest phrase boundary (4 bars or 8 bars)

### 2.4 — Choose the Transition Type

Based on the compatibility analysis between Track A and Track B, select one of the following transition types:

```

│  DECISION MATRIX                                                  │

│  Condition         │  BPM Diff  │  Transition Type               │

│  Same key region   │  ≤ 4 BPM   │  LONG BLEND (16-32 bars)       │

│  + compatible key  │            │  Full frequency layering       │

│                    │            │  with EQ swapping              │

│  Same key region   │  5-10 BPM  │  MEDIUM BLEND (8-16 bars)      │

│  + compatible key  │            │  Tempo sync via time-stretch   │

│                    │            │  + EQ swap                     │

│  Clashing keys     │  ≤ 4 BPM   │  SHORT BLEND (4-8 bars)        │

│                    │            │  Percussion-only overlap       │

│                    │            │  (filter out melodic content)   │

│  Clashing keys     │  5-10 BPM  │  DROP MIX (2-4 beats)          │

│                    │            │  Echo/reverb out Track A       │

│                    │            │  Hard cut to Track B on beat 1 │

│  Any keys          │  > 10 BPM  │  ECHO OUT + COLD START         │

│                    │            │  Fade Track A with echo tail   │

│                    │            │  Brief silence (200-500ms)     │

│                    │            │  Start Track B cleanly         │

│  Track flagged     │  Any       │  SIMPLE CROSSFADE (5-8 sec)    │

│  “not_mixable”     │            │  Linear or equal-power volume  │

│                    │            │  fade only                     │

```

### 2.5 — Vocal Collision Avoidance

**Instructions:**

- Before finalizing the blend window, cross-reference the **vocal activity timelines** of both tracks

- If both Track A and Track B have active vocals during the proposed overlap:

- **Option A:** Shorten the blend window so it ends before Track B’s vocals begin

- **Option B:** Shift the mix_in_point of Track B later (skip its intro if it has vocals) and start at a non-vocal section

- **Option C:** If shortening is impossible, apply a mid-frequency cut (see Phase 3) to Track A’s vocal range during the overlap to duck the outgoing vocals beneath the incoming vocals

- **Rule: Never allow two vocal lines to compete for more than 2 bars.** If you cannot resolve the collision, fall back to a DROP MIX or ECHO OUT transition.

## PHASE 3: REAL-TIME AUDIO PROCESSING

This phase describes the actual DSP operations you must apply to the audio streams during the transition window.

### 3.1 — Tempo Synchronization (Time-Stretching)

**When:** BPM difference between Track A and Track B is between 1 and 10 BPM.

**Instructions:**

- Choose a target BPM for the overlap period. Two strategies:

- **Split the difference:** Target BPM = average of both BPMs

- **Favor Track B:** Gradually shift Track A’s tempo toward Track B’s BPM over the blend, so Track B plays at its native tempo

- Apply **pitch-preserving time-stretching** to the track(s) that need adjustment:

- Use a phase vocoder with identity phase locking for best quality

- Alternatively, use WSOLA (Waveform Similarity Overlap-Add) for lower latency

- The stretch ratio = target_bpm / original_bpm

- **Beat alignment:** After time-stretching, align the beat grids. Ensure that the downbeats of Track A and Track B land simultaneously (within ±10ms tolerance). If they drift, apply micro-corrections each bar.

**Quality Notes:**

- Time-stretching artifacts become audible beyond ±6% stretch (roughly ±8 BPM at 120 BPM). If the required stretch exceeds this, prefer a DROP MIX or ECHO OUT to avoid artifacts.

- Apply stretch to the track with fewer exposed vocals if possible (stretching vocals is more audibly artifacted than stretching drums/instruments).

### 3.2 — Multi-Band Frequency Crossfade (EQ Swap)

**When:** LONG BLEND or MEDIUM BLEND transition types.

**Objective:** Prevent low-frequency overload and vocal collisions by independently crossfading different frequency bands.

**Instructions:**

- Split both audio streams into 3 bands using Linkwitz-Riley crossover filters (24dB/octave for clean separation):

- **Low band:** 20Hz – 180Hz

- **Mid band:** 180Hz – 4kHz

- **High band:** 4kHz – 20kHz

- Apply the following gain curves over the blend duration:

```

TIME →  [Blend Start]                    [Blend Midpoint]                   [Blend End]

0%                               50%                               100%

TRACK A — LOW BAND:

████████████████████████████████  ████████████▓▓▓▓▒▒░░            (drops to 0)

Full gain ──────────────────────→ Cuts sharply at midpoint beat ──→ Silent

TRACK A — MID BAND:

████████████████████████████████  ████████▓▓▓▓▒▒▒▒░░░░░░         (gradual fade)

Full gain ──────────────────────→ Smooth logarithmic fade ───────→ Silent

TRACK A — HIGH BAND:

████████████████████████████████  ██████████████████▓▓▓▓▒▒▒░░░░  (slow fade)

Full gain ──────────────────────→ Slow fade (highs can coexist) ─→ Silent

TRACK B — LOW BAND:

░░▒▒▓▓████████████████████████  (enters at midpoint)

Silent ─────────────────────────→ Cuts in sharply at midpoint ───→ Full gain

TRACK B — MID BAND:

░░░░░░▒▒▒▒▓▓▓▓████████████████████████████████  (gradual rise)

Silent ─────────→ Smooth logarithmic rise ──────────────────────→ Full gain

TRACK B — HIGH BAND:

░░░░▒▒▒▓▓▓████████████████████████████████████████████████  (early entry)

Silent → Rises early (highs can coexist) ──────────────────────→ Full gain

```

**Critical rule for the LOW BAND:**

- The low bands of Track A and Track B should **never both be at full gain simultaneously**. At the swap point (midpoint), cut Track A’s low band and bring in Track B’s low band on a **downbeat**. This is called the **bass swap**. It should happen instantly (within 1 beat) to avoid the muddy, phase-cancelled sound of two basslines overlapping.

- The bass swap point must be quantized to beat 1 of a bar.

### 3.3 — Gain Compensation During Overlap

**Objective:** Two tracks playing simultaneously are louder than one. You must compensate to avoid clipping and perceived volume spikes.

**Instructions:**

- During the overlap, apply a combined gain reduction using this formula:

```

Gain_reduction_dB = -3.0 * overlap_intensity

```

Where `overlap_intensity` is a value from 0.0 to 1.0 representing how much energy is overlapping (based on the combined RMS of both signals relative to a single track’s RMS).

- In practice: at peak overlap (both tracks playing at significant levels), reduce the master output by approximately **-2.5 to -3.5 dB**

- Use a limiter on the master bus with the following settings:

- Threshold: -1.0 dBFS

- Attack: 0.1 ms

- Release: 50 ms

- Ratio: ∞:1 (brick wall)

- This prevents any transient peaks from clipping during the transition

### 3.4 — Echo / Reverb Tail for Short Transitions

**When:** DROP MIX or ECHO OUT transition types.

**Instructions:**

- When Track A must exit abruptly (e.g., due to key clash or large BPM gap):

1. At the mix_out_point, stop feeding new audio from Track A into the main bus

2. Apply a **feedback delay** to Track A’s last 1–2 beats:

- Delay time: Sync to Track B’s beat grid (e.g., ¼ note or 1/8 note at Track B’s BPM)

- Feedback: 40–60% (so the echo repeats 3–5 times before decaying)

- Apply a high-pass filter to the delay line at 400Hz (so the echo doesn’t muddy the bass of Track B)

3. Simultaneously apply a **reverb tail**:

- Decay time: 2–4 seconds

- Pre-delay: 20ms

- Wet/dry: 100% wet (no dry signal, just the tail)

- Apply a low-pass filter at 8kHz to the reverb return (softens the tail)

4. While the echo/reverb decays, bring in Track B at full volume on beat 1

- This creates the illusion that Track A “dissolves into space” while Track B takes over cleanly.

### 3.5 — High-Pass Filter Sweep (Alternative Transition Effect)

**When:** Can substitute for or complement the echo-out technique.

**Instructions:**

- Over the last 4–8 bars of Track A, apply a resonant high-pass filter that sweeps upward:

- Start frequency: 20Hz (fully open, no filtering)

- End frequency: 2000–5000Hz (only the high frequencies remain)

- Sweep curve: Exponential (slow at first, accelerating toward the end)

- Resonance (Q): 1.5–3.0 (adds a slight “whine” at the filter frequency, mimicking a DJ filter sweep)

- As the filter removes the lows and mids from Track A, bring in Track B underneath with its full frequency spectrum

- At the final beat, cut Track A entirely

## PHASE 4: SEQUENCING AND QUEUE INTELLIGENCE (OPTIONAL ENHANCEMENT)

If you want your Automix to go beyond per-transition processing and optimize the entire queue:

### 4.1 — Queue Reordering for Harmonic Flow

- When the user has a queue of N songs, analyze all pairwise key compatibilities

- Use a nearest-neighbor algorithm on the Camelot Wheel to find the optimal playback order that minimizes harmonic clashes

- Present the reordered queue as a suggestion (do not force it; let the user accept or reject)

### 4.2 — Energy Arc Management

- Compute an “energy score” per track (based on BPM, loudness, and spectral brightness)

- When auto-generating a playlist or DJ set, arrange tracks to create an energy arc:

- Start low energy

- Build to a peak at ~60-70% through the set

- Bring it back down toward the end

- This mimics how professional DJs structure their sets

### 4.3 — Genre Awareness

- Tag tracks by genre/subgenre

- Apply genre-specific transition rules:

- **EDM / House / Techno:** Long blends (16-32 bars), full beat-matching, bass swaps on drops

- **Hip-Hop / R&B:** Shorter blends (4-8 bars), echo-out preferred, avoid beat-matching if BPM varies

- **Pop / Indie:** Medium blends (8-16 bars), prioritize vocal-free zones

- **Classical / Jazz / Acoustic:** Simple crossfade only; do not beat-match

## SUMMARY: COMPLETE TRANSITION EXECUTION FLOW

```

│                        AUTOMIX EXECUTION PIPELINE                       │

│                                                                         │

│  1. Track A approaches its final segments                               │

│     │                                                                   │

│  2. Load pre-analyzed metadata for Track A and Track B                  │

│     ├── BPM, beat grid, key, structure, loudness, vocal activity        │

│     │                                                                   │

│  3. Determine mix_out_point (Track A) — snap to downbeat                │

│     │                                                                   │

│  4. Determine mix_in_point (Track B) — snap to downbeat                 │

│     │                                                                   │

│  5. Check vocal collision → adjust if needed                            │

│     │                                                                   │

│  6. Compute BPM difference and key compatibility                        │

│     │                                                                   │

│  7. Select transition type from decision matrix                         │

│     │                                                                   │

│  8. Calculate blend duration → round to nearest phrase                  │

│     │                                                                   │

│  9. BEGIN REAL-TIME PROCESSING:                                         │

│     ├── Start time-stretching (if needed) → align beat grids            │

│     ├── Begin multi-band crossfade:                                     │

│     │   ├── Highs of Track B enter first                                │

│     │   ├── Mids crossfade gradually                                    │

│     │   └── Bass swaps cleanly on a downbeat                            │

│     ├── Apply gain compensation (-3dB at peak overlap)                  │

│     ├── Apply echo/reverb if short transition                           │

│     └── Apply HPF sweep if needed                                       │

│     │                                                                   │

│ 10. Track A fully silent → Track B now playing solo at full gain        │

│     │                                                                   │

│ 11. Begin pre-analyzing Track C (next in queue)                         │

│                                                                         │

```

## RECOMMENDED LIBRARIES AND TOOLS

| Task | Recommended Tools |

|:---|:---|

| Beat/BPM detection | `madmom`, `librosa`, `essentia`, custom CNN |

| Key detection | `librosa.feature.chroma`, `essentia`, `keyfinder` |

| Structural segmentation | `MSAF`, `librosa`, custom CRNN on SALAMI dataset |

| Source separation (vocals) | `Demucs` (Meta), `Spleeter` (Deezer) |

| Time-stretching | `Rubber Band Library`, `SoundTouch`, `librosa.effects.time_stretch` |

| EQ / Filtering | `scipy.signal` (IIR/FIR), `biquad` filters, or real-time DSP frameworks |

| Reverb / Delay | `pedalboard` (Spotify), custom FDN reverb, or convolution reverb |

| Loudness metering | `pyloudnorm`, `ffmpeg` (EBU R128), `libebur128` |

| Real-time audio engine | `PortAudio`, `JUCE`, `AudioKit`, `Oboe` (Android), `AVAudioEngine` (Apple) |

## FINAL NOTES

- **Latency budget:** Pre-analyze tracks during buffering/download. The transition decision engine should execute at least **10 seconds before** the mix_out_point so all DSP operations can be prepared.

- **User controls:** Expose a toggle for “Automix On/Off” and optionally a slider for “Transition Length Preference” (Short / Medium / Long).

- **Graceful degradation:** If any analysis step fails or returns low-confidence data, always fall back to a simple equal-power crossfade rather than producing a broken mix.

- **Testing:** Build a validation suite with known song pairs that cover all transition types. Listen-test every update. Automated metrics (spectral discontinuity, loudness deviation, beat alignment error) are helpful but human ears are the final judge.